# Automation of Concrete Mix Design

Data Report

Group M

Ava Johnson, Ayub Abdi, Jacob Lamoureux, Noah Morland

Department of Civil & Environmental Engineering, University of Nebraska-Lincoln

CIVE 202: Civil Engineering Analysis II

Dr. Kaycie Lane

February 24, 2026

Contents

Introduction	3

Methods	3

Inputs users must enter	4

Outputs the script produces	4

Results & Discussion	4

Sample Mix Designs and Result Comparison	5

References	6

# Introduction

The Nebraska Department of Transportation (NDOT) has tasked our team with taking its current concrete mix design spreadsheet and turning it into a working Python program. NDOT currently uses an Excel file to walk through and calculate each required input such as the cement content, water-content ratio, air content, and aggregates percentages needed for one cubic yard of concrete, but the process is manual and can easily lead to mistakes. By moving these calculations to Python, NDOT will have a more consistent program that is easily replicable for future uses. (NDOT, 2017a)

For this project, our team recreated the formulas from the NDOT Mix Design Workbook in Python and built a script that asks the user for inputs step-by-step the same way the Excel sheet does. The program then prints a clean weight chart showing all the material amounts. To check that the program works correctly, we also researched four realistic concrete mix scenarios using NDOT specifications and NDOT’s research on aggregate gradation (NDOT, 2019). Running these scenarios through the model helped confirm that our Python version follows the same logic NDOT uses in its official mix design process.

# Methods

First, the Mix Design worksheet was reviewed to find the constants, the inputs, and the outputs the sheet calculates. Those were brought into a Python script, so the code does the same steps in the same order with the same units. Two fixed numbers were set at the top of the script, so all conversions are consistent: 27 (this is the number of cubic feet in one cubic yard) and 62.4 (this is the unit weight of water). Specific gravity is a dimensionless ratio compared to water; multiplying a material’s specific gravity by 62.4 gives that material weight per cubic foot (NDOT, n.d.).

Each Excel calculation was then turned into a small Python function with a clear purpose and a matching letter from the Excel sheet, so it is easy to cross reference. A function was defined for water weight (calc_water_weight_Q) that takes the cementitious weights and the water-cement ratio and returns the water required- in pounds. Separate functions were defined for the volumes of each cementitious material (calc_cement_volume_R, calc_fly_ash_volume_S, calc_silica_volume_T, calc_other_SCM_volume_U); each one converts a material’s weight and specific gravity into its volume in ft³. A function was defined for air volume (calc_air_volume_V) that turns the target air percentage into ft³ for one yd³. A function was defined for water volume (calc_volume_water_W) that converts the water weight into ft³. A function was defined for remaining aggregate volume (calc_total_agg_volume_X) that subtracts known volumes to determine remaining volume, which will be aggregate.. Finally, three functions were defined for the aggregate weights (calc_fine_agg_weight_Y, calc_coarse_agg_weight_Z, calc_other_agg_weight_AA). These take the remaining aggregate volume, the chosen percent split, and the corresponding specific gravities and return the fine, coarse, and other aggregate weights in pounds (NDOT, n.d.).

After the functions were in place, simple prompts were added so a user can type the same numbers shown in the worksheet. The script then runs the functions in the same sequence the worksheet uses: water weight first, then the four cementitious volumes, then air and water volumes, then total aggregate volume, and finally the three aggregate weights. The script prints a one cubic yard summary with the same labels as the worksheet, so the two can be compared line by line (NDOT, n.d.).

Inputs users must enter 
• Project ID
• Concrete class name
• Weights of each cementitious material in lb/yd³ (Cement, fly ash, silica, and other SCM)
• Water-cementitious ratio by weight (unitless number like 0.42)
• target air as a percentage (enter 6 for 6%, not 0.06)
• Target aggregate splits as percents of the total aggregate volume (fine aggregate, coarse aggregate, and other aggregate)
• Specific gravities for each material (unitless)

Outputs the script produces
• Q: water weight in pounds
• R, S, T, U: volumes of cement, fly ash, silica fume, and other SCM in ft³
• V: entrained air volume in ft³ for one yd³
• W: water volume in ft³
• X: total aggregate volume in ft³
• Y, Z, AA: fine, coarse, and other aggregate weights in pounds

# Results & Discussion

To check whether our Python model accurately followed the NDOT’s mix design process, we tested it using four different concrete mix scenarios. Each scenario used realistic values for cementitious materials, water–cement ratio, air content, and aggregate percentages based on NDOT’s Mix Design Submittal Workbook (NDOT, 2017a) and the NDOT Standard Specifications for Highway Construction (NDOT, 2017b). Running these mixes through the program allowed us to compare how changes in the inputs affected the final material weights. It also proved that the automated system successfully replicates the NDOT Excel workflow while offering a more transparent and repeatable calculation of concrete mix designs.

## Sample Mix Designs and Result Comparison

Mix 1 made a good baseline comparison. It used a typical water-cement ratio and air content which produced a balanced distribution of fine and coarse aggregate and a reasonable water demand. These results required a standard amount of supplementary cementitious materials (SCM) replacement that is typically used to enhance the concrete durability. This mix closely resembles NDOT’s Class 47B pavement concrete, which is commonly used for roads (NDOT, 2017, Section 1002, Table 1002.02). The final weight chart showed no extreme values.

Mix 2 used a higher water-cement ratio and lower cement content. As expected, this produced a higher water weight, slightly reduced aggregate weights, and prompted slightly reduced SCM use. This type of mix is typically used where strength requirements are lower, such as sidewalks and driveways (NDOT, 2017).

Mix 3 included a higher percentage of SCM, especially fly ash. This shifted the cementitious volume distribution and slightly reduced the water demand but maintained a standard air content. The Python model showed that the total cementitious volume changes based on the specific gravity of the SCMs, which is consistent with NDOT’s research on optimized aggregate gradations (Nebraska Department of Roads, 2002). This mix is often used when long-term durability is desired due to SCMs improving workability. This includes cases like bridge approaches and designs that require long-term performance.

Mix 4 used a low water-cement ratio, high cement content, and standard SCM amounts. A lower water weight and higher cementitious volume reduced the available volume of aggregates. This behavior is to be expected and matched NDOT’s specifications for higher-strength concrete classes (NDOT, 2017, Section 1002, Table 1002.02). This mix design is required to achieve better durability and early strength which is typically desired in structures where performance is critical to ensure safety, such as structural components and bridge decks.

Overall, the differences amongst the four mixes clearly showed that changes in the input parameter trends matched NDOT’s established mix design behavior (NDOT, 2017; Nebraska Department of Transportation, 2018). Higher air contents reduced aggregate weights, higher SCM contents shifted the cementitious balance, and lower water-cement ratios increased cement demand. The tests demonstrated that the program is reliable, consistent, and capable of handling a wide range of realistic concrete designs.

References

Nebraska Department of Roads. (2002, September). Optimized aggregate gradations for Portland cement concrete mix designs evaluation (Final Report P336). Nebraska Department of Roads.
https://dot.nebraska.gov/media/3isfdv45/final-report-p336.pdf

Nebraska Department of Transportation. (n.d.). Mix design submittal worksheet [Microsoft Excel spreadsheet]. Nebraska Department of Transportation.
https://dot.nebraska.gov/media/jp3paote/mix-design-submittal.xlsx

Nebraska Department of Transportation. (2017). Standard specifications for highway construction (2017 ed.). Nebraska Department of Transportation.
https://dot.nebraska.gov/media/msobntqs/2017_spec_book.pdf

Nebraska Department of Transportation. (2018). Pavement design manual. Nebraska Department of Transportation.

Nebraska Department of Transportation. (2017). Standard specifications for highway construction (Section 1002, Table 1002.02; Section 1009). Nebraska Department of Transportation.